| A3 Report — Predictive maintenance of a milling machine Continuous improvement · TPM · DMAIC · AI4I 2020 dataset (UCI #601, 10,000 cycles) | Esteban López Industrial Eng. 2026 |
| --- | --- |

1.  Background & problem  (DEFINE)

Unplanned breakdowns are the main enemy of availability — the first pillar of OEE. Reactive maintenance stops the line at the most expensive moment. Using 10,000 operating cycles of a milling machine (6 process variables, a machine-failure label and 5 failure modes), this A3 builds a predictive-maintenance strategy that converts unplanned stops into planned interventions — the core idea of TPM.

Goal: predict failures early, prioritise the failure modes that matter, and lift availability.

2.  Current state  (MEASURE)

| 10,000 operating cycles | 3.39% failure rate (339 cycles) | ≈30 cycles MTBF | 0 missing values |
| --- | --- | --- | --- |

Failures are rare and imbalanced. A Pareto of the five failure modes shows where reliability effort pays off:

Failure modes — count and cumulative

| Failure mode | Occurrences | % of total | Cumulative % |
| --- | --- | --- | --- |
| HDF — Heat dissipation | 115 | 30.8% | 30.8% |
| OSF — Overstrain | 98 | 26.3% | 57.1% |
| PWF — Power | 95 | 25.5% | 82.6% |
| TWF — Tool wear | 46 | 12.3% | 94.9% |
| RNF — Random | 19 | 5.1% | 100.0% |

Note: the 5 modes total 373 occurrences across 339 failed cycles (some cycles trigger more than one mode).

3.  Root-cause analysis  (ANALYZE)

The three vital-few modes — heat dissipation (HDF), overstrain (OSF) and power (PWF) — concentrate 82.6% of all failure occurrences and share one physical root: torque, rotational speed and the air–process temperature gap. Rather than feeding raw sensors blindly, the model uses features derived from that physics:

Mechanical power = Torque × angular speed → drives power failures

Temperature gap = Process − Air temperature → drives heat-dissipation failures

Tool wear × Torque → proxy for overstrain failures

Ishikawa — cause categories (6M)

Machine: roll/tool wear, bearings

Material: workpiece hardness, tool grade

Method: feed rate, spindle speed, cooling

Measurement: sensor calibration, thresholds

Manpower: setup variability

Environment: ambient temperature

4.  Proposed improvement  (IMPROVE)

A Random Forest predicts the machine-failure label from the process variables plus the engineered physics. Key engineering judgement: with only 3.4% failures, accuracy is misleading (a "never fails" model already scores ~96.6%). The model is judged on RECALL of the failure class — the share of real breakdowns caught in time.

| Aspect | Detail |
| --- | --- |
| Algorithm | Random Forest (400 trees), class_weight balanced |
| Validation | Stratified 25% hold-out |
| Judged on | Recall, F1, ROC-AUC, PR-AUC (not accuracy) |
| Operating lever | Decision-threshold tuning: recall vs. false alarms |

*The reproducible notebook computes the exact metrics, confusion matrix and feature importance on the full data.

5.  Action plan & control  (CONTROL)

Predict, don’t react — each flagged failure becomes a planned stop; direct lift to availability/OEE.

Prioritise the vital-few modes — SPC limits on temperature gap, torque and mechanical power.

Tune the threshold — trade false alarms for higher recall based on breakdown vs. check cost.

Condition-based alarms — turn top features into shop-floor thresholds (poka-yoke backup).

PDCA — track MTBF and recall monthly; retrain as tooling changes.

Data source: AI4I 2020 Predictive Maintenance — UCI Machine Learning Repository (#601). Analysis and model reproducible in the attached notebook.