| Technical Report — Gas-turbine engine (Brayton cycle) Thermodynamics · Gas turbine & jet propulsion · Cold-air-standard | Esteban López Industrial Eng. 2026 |
| --- | --- |

1.  Problem & specification

Analyse the air-standard Brayton cycle behind every gas turbine and jet engine. Compute each state, the thermal efficiency (ideal and with real compressor/turbine losses), the back-work ratio, and find the pressure ratio that maximises net work for a given turbine-inlet temperature.

| rp = 12 pressure ratio | 1400 K turbine inlet (TIT) | 0.85 / 0.88 comp. / turb. η | 288 K inlet temperature |
| --- | --- | --- | --- |

2.  Method & results

States: 1 = compressor inlet, 2 = compressor exit, 3 = turbine inlet, 4 = turbine exit. Cold-air-standard (cp = 1.005 kJ/kg·K, k = 1.4); real compression/expansion via isentropic efficiencies.

| Quantity | Value |
| --- | --- |
| Ideal thermal efficiency | 50.8 % |
| Real thermal efficiency | 36.2 % |
| Net work | 277 kJ/kg |
| Turbine work | 629 kJ/kg |
| Compressor work | 352 kJ/kg |
| Heat input q_in | 765 kJ/kg |
| Back-work ratio | 56 % |
| Compressor exit T2 / Turbine exit T4 | 638 K / 774 K |

3.  Optimum pressure ratio & back-work

Efficiency rises with pressure ratio, but net work peaks at an optimum rp (≈ 10–12 here). The compressor consumes ~56 % of the turbine output — far more than the <1 % pump work of a steam cycle:

4.  Aerospace link — the turbojet

A turbojet is an open Brayton cycle: the turbine only drives the compressor, and the remaining energy is left in the fast exhaust to produce thrust, F = ṁ·(V_exit − V_inlet) + (p_e − p_0)·A_e. The same analysis sizes aircraft engines, auxiliary power units (APUs) and ground/marine gas turbines — a direct bridge from industrial energy systems to propulsion and aerospace, where availability, integrity and continuity (as in GNSS) are the equivalent figures of merit.

5.  Conclusion

Pick the pressure ratio — net work peaks at an optimum rp for the chosen TIT.

Chase turbine-inlet temperature — the dominant lever; hence materials and blade cooling.

Mind the back-work ratio — 56 % explains the Brayton vs Rankine difference.

Extend to real engines — regeneration, intercooling, reheat, and combined Brayton-Rankine plants.

Cold-air-standard Brayton analysis. Fully reproducible in the attached notebook.