| A3 Report — Line balancing of a packaging line Lean line balancing · Discrete-event simulation (SimPy) · DMAIC | Esteban López Industrial Eng. 2026 |
| --- | --- |

1.  Background & problem  (DEFINE)

Most shop-floor improvement must happen on site, so this A3 demonstrates the method on a digital twin. A 6-station packaging/conditioning line must deliver 80 units/hour, which sets a takt time of 45 s/unit. If any single station exceeds takt it throttles the whole line and demand is missed. Using a SimPy discrete-event model, we locate the bottleneck and quantify the gain from balancing the line — before touching anything physical.

Goal: find the bottleneck, balance the line, and measure the improvement in throughput, lead time and WIP.

2.  Current state  (MEASURE)

| 45 s takt time (80 u/h) | S3 = 55 s bottleneck (> takt) | 1.22 S3 utilisation | ~65 u/h capped throughput |
| --- | --- | --- | --- |

Station cycle times against takt — station S3 sits above the line and caps the whole process:

3.  Analysis  (ANALYZE)

Utilisation at demand equals cycle time / takt. Station S3 = 55/45 = 1.22 (> 1): it cannot keep up, so the line is capped at ~65 u/h, below the 80 u/h demand. The remaining stations are under-used and idle waiting on S3 — classic line imbalance. The fix is to redistribute work so no station exceeds takt.

4.  Proposed improvement  (IMPROVE)

Rebalance ~13 s of S3 work to lighter stations, keeping the same total work content (221 s). Every station then sits below takt:

| Metric | Before | After |
| --- | --- | --- |
| Bottleneck cycle time | 55 s (S3) | 42 s |
| Throughput | ~65 u/h | ~86 u/h (+31%) |
| Mean lead time | backlog grows | ≈27 min |
| Average WIP | unbounded | ≈37 units |
| Demand (80 u/h) met? | No | Yes |

The gain comes purely from balancing — no extra labour.

5.  Action plan & control  (CONTROL)

Standardised work — keep station cycle times near the balanced target.

Visual takt — react when a station drifts above it (the new bottleneck).

Re-balance on demand change — takt is set by demand; recompute when the order rate moves.

Attack variability — high variability inflates WIP and lead time even under takt.

PDCA by simulation — test every change on the model before touching the physical line.

Method: discrete-event simulation in SimPy with realistic line parameters. Fully reproducible in the attached notebook.