| A3 Report — Quality control on a Bosch assembly line Continuous improvement · Six Sigma · DMAIC · Bosch Production Line (Kaggle, 14.3 GB) | Esteban López Industrial Eng. 2026 |
| --- | --- |

1.  Background & problem  (DEFINE)

Bosch records thousands of measurements as each part flows through its assembly line. The goal: predict which parts will fail quality control so they can be caught early. The data is real and large-scale — 1,184,687 parts, 968 numeric measurements across 51 stations on 4 production lines (14.3 GB) — and extremely imbalanced, so it challenges both the statistics and the engineering of the analysis.

Goal: map the value stream, find where defects concentrate, and build a defect-prediction model with the right metric.

2.  Current state  (MEASURE)

| 1,184,687 parts | 0.58% defect rate (6,879) | 968 sensors · 51 stations | 14.3 GB dataset size |
| --- | --- | --- | --- |

The feature names L{line}_S{station}_F{feature} act as a value-stream map. A Pareto of measurement density shows where the line is most complex and instrumented:

Measurement density per production line

| Production line | Sensors | % of total | Cumulative % |
| --- | --- | --- | --- |
| L1 — core processing | 513 | 53.0% | 53.0% |
| L3 — finishing | 245 | 25.3% | 78.3% |
| L0 — entry | 168 | 17.4% | 95.7% |
| L2 — short branch | 42 | 4.3% | 100.0% |

3.  Root-cause analysis  (ANALYZE)

Lines L1 and L3 carry 78% of all measurements — the most complex, most-monitored stages, and the natural place to focus SPC. Because the date features timestamp each step, every part’s route through the stations can be reconstructed; a station-level defect rate (computed in the notebook) reveals where failing parts concentrate.

Engineering challenges and how they were handled

Extreme imbalance — only 0.58% defects, so MCC (not accuracy) is the metric.

Volume (14.3 GB) — stream the file in chunks and down-cast to float32.

Sparsity — each part visits only some stations, so most features are NaN.

4.  Proposed improvement  (IMPROVE)

A HistGradientBoosting model predicts the failure label. It handles NaN natively and scales to millions of rows. Engineering judgement: accuracy is meaningless here (a "all pass" model scores 99.4%); the model is judged on the Matthews Correlation Coefficient (MCC) and the decision threshold is tuned to maximise it.

| Aspect | Detail |
| --- | --- |
| Algorithm | HistGradientBoosting (native NaN, class_weight balanced) |
| Data handling | chunked read + float32 down-cast; majority down-sampling |
| Metric | MCC + threshold tuning (not accuracy) |
| Expected MCC* | ~0.40–0.48 single model (winning ensembles ~0.52) |

*The reproducible notebook computes the exact MCC, threshold curve and confusion matrix on the full data.

5.  Action plan & control  (CONTROL)

Focus SPC where the process is complex — highest-defect-rate stations on lines L1 and L3 first.

Catch escapes, accept some false alarms — tune threshold to MCC / cost of a missed defect.

Trace every failing part’s route — recurring station sequences point to the stage to fix.

Poka-yoke at critical stations — turn top signals into in-line gates that stop defects.

PDCA at scale — re-score MCC and the station defect map per shift; retrain as the line drifts.

Data source: Bosch Production Line Performance — Kaggle competition (2016). Analysis and model reproducible in the attached notebook.